# MATLAB Solver Notes

Differential Algebraic Equations (DAEs)

- https://www.mathworks.com/help/matlab/math/solve-differential-algebraic-equations-daes.html

- Ode15s and ode23t can handle index-1 linearly implicit problems

  - Singular mass matrix

  - Solver detects the singular mass matrix

- Ode15i can solve general DAEs in fully implicit form

  - Singular Jacobian matrix

Matrix preconditioning

Experiment Design

# Literature Review

## Gradient Calculation

Sozer, Emre, Christoph Brehm, and Cetin C. Kiris. “Gradient Calculation Methods on Arbitrary Polyhedral Unstructured Meshes for Cell-Centered CFD Solvers.” In 52nd Aerospace Sciences Meeting. National Harbor, Maryland: American Institute of Aeronautics and Astronautics, 2014. https://doi.org/10.2514/6.2014-1440.

- Ideal gradient operator choice depends on type of cells used (shape)

- “the price paid (for ease of meshing) in return is often understood as reduced computational efficiency for a given mesh size, accuracy of the simulations may also suffer.”

- “In short, giving up the great level of user involvement and control over the mesh generation process is desirable in terms of the cost of mesh generation, but the burden is transferred to the unstructured solver to maintain accuracy and stability despite potentially sub-quality meshes.”

- Least squares and green gauss methods work similarly for regular meshes, but LSQ was more tolerant to mesh distoritions

  - LSQ accuracy breaks down for highly stretched meshes with curvature

- Hybrid methods using least squares in most regions, but Gauss-green in thin, distorted regions

- Inverse weighted regression method best for irregular meshes according to Correa et al.

## Method of Moments

Yuan, C., F. Laurent, and R.O. Fox. “An Extended Quadrature Method of Moments for Population Balance Equations.” Journal of Aerosol Science 51 (September 2012): 1–23. https://doi.org/10.1016/j.jaerosci.2012.04.003.

Notes:

- Quadrature-based moment methods (QBMM) are an important class of methods for which the accuracy of the solution can be improved in a controlled manner by increasing the number of quadrature nodes.

Chapter 7. Statistical Estimation (Stanford)

- First sample moment is mean

- Second sample moment is variance

- iid = independent and identically distributed variables/samples

Dürr, Robert, and Andreas Bück. “Approximate Moment Methods for Population Balance Equations in Particulate and Bioengineering Processes.” Processes 8, no. 4 (April 1, 2020): 414. https://doi.org/10.3390/pr8040414.

Notes:

- In moment methods, the dynamics of integral quantities – such  as the mean or standard deviation – are computed instead of considering a complete number density function.

- In general, a closed set of moment equations is rarely found and, therefore, an approximate closure is necessary (i.e., a sound approximation of unknown moments by known ones).

- Method of Moments with Interpolating Closure

- Quadrature Method of Moments

    - Due to its balance in favorable properties such as robustness, accuracy, and relatively low computational effort, QMoM has found wide application for efficient solution of PBEs emerging from modeling of spatially distributed polydisperse gas–liquid-particle flows

  - Direct Quadrature Method of Moments (DQMOM)

    - One of the drawbacks of (D)QMoM is that increasing accuracy in terms of a higher amount of abscissas can yield ill-conditioned moment-inversion and thereby numerical instability and failure of the overall approximate moment method.

  - Extended Quadrature Method of Moments (promising)

    - Explicit form of the NDF

    - Furthermore, the number of quadrature nodes can be increased independently from the number of transported moments and thereby achieve a more accurate representation of non-closing source terms

    - Can use any type of distribution (Gaussian, Gamma, Beta, Laplace, Log-normal, Weibull)

  - Sectional Quadrature Method of Moments (promising)

    - Particle property space

  - Maximum Entropy (ME) Approach

    - More complex numerical effort, but allowes more accurate reconstruction of the NDF

  - Moment Projection Method

    - Other methods fail when shrinkage dominates – abs

  - Multivariate systems

    - Unknown moments can be represented with interpolation

    - QMoM and principal component analysis

  - Abscissas are the discrete points used to approximate the distribution

Shiea, Mohsen, Antonio Buffo, Marco Vanni, and Daniele Marchisio. “Numerical Methods for the Solution of Population Balance Equations Coupled with Computational Fluid Dynamics.” Annual Review of Chemical and Biomolecular Engineering 11, no. 1 (June 8, 2020): 339–66. https://doi.org/10.1146/annurev-chembioeng-092319-075814.

Notes:

- Eulerian CFD

  - Key factors for selecting appropriate Eulerian methods for multiphase CFD: Stokes number and volume fraction of disperse phase elements

  - Dusty gas model

    - Small particles, low Stokes number, velocity just matches continuous phase

  - Mixture model

    - Velocity of particles differs from liquid because of higher Stokes.

    - Under equilibrium assumption, the relative velocity between the disperse elements and the continuous phase can be expressed as an algebraic relation obtained from a force balance on the elements (usually drag and buoyancy forces).

    - disperse system is dilute enough, one momentum balance equation is solved for the velocity of the continuous phase, and then the algebraic relation is used to calculate the velocity of the disperse phase

  - Two and multifluid models

Attarakih, Menwer M., Christian Drumm, and Hans-Jörg Bart. “Solution of the Population Balance Equation Using the Sectional Quadrature Method of Moments (SQMOM).” Chemical Engineering Science 64, no. 4 (February 2009): 742–52. https://doi.org/10.1016/j.ces.2008.05.006.

Notes:

Marchisio, Daniele L., R.Dennis Vigil, and Rodney O. Fox. “Quadrature Method of Moments for Aggregation–Breakage Processes.” Journal of Colloid and Interface Science 258, no. 2 (February 2003): 322–34. https://doi.org/10.1016/S0021-9797(02)00054-1.

Notes:

- To apply QMOM, the general aggregation-breakage PBE must be written in terms of the moments of the number density function

  - Moment transformation:

    - mkt=0∞nL;tLkdL

    - ∂nL;t∂t=BaL;t-DaL;t+BbL;t-DbL;t

    - dmktdt=Bka-Dka+Bkb-Dkb

    - The B and D terms represent source and sink terms related to aggregation (a) and breakage (b). They are integral equations listed in the reference.

- Application of QMOM

  - Quadrature approximation: mk= 0∞nLLkdL≈i=1NwiLik

  - Where wi are weights, and Li is the abscissas. They are determined using the product differene algorithm from the lower order moments.

  - Details of algorithm in appendix B

  - Complete derivation in three sources

  - “Previous results [14,15] showed that a quadrature approximation with three nodes (N = 3) is sufficient to describe moment evolutions with good accuracy.”

- Many breakage and aggregation kernels

  - Kernel is homogeneous if it scales as: βaL, aλ=aγβL,λ    a→∞

  - γ is the homogeneity index – determines the asymptotic behavior of the system.

Marchisio, Daniele L., Jesse T. Pikturna, Rodney O. Fox, R. Dennis Vigil, and Antonello A. Barresi. “Quadrature Method of Moments for Population‐balance Equations.” AIChE Journal 49, no. 5 (May 2003): 1266–76. https://doi.org/10.1002/aic.690490517.

Notes:

- Standard moment method

- Quadrature Mehtod of Moments

  - Closure problem is solved using a quadrature approx..

McGraw, Robert. “Description of Aerosol Dynamics by the Quadrature Method of Moments.” Aerosol Science and Technology 27, no. 2 (January 1997): 255–65. https://doi.org/10.1080/02786829708965471.

Notes:

- Replace the dynamical equations for moment evolution with a quadrature based approximate set that satisfies closure under a broader range of conditions.

- Closure of the equations is the main challenge for the standard method of moments

- Quadrature Method of Moments (QMOM)

  - Moments of f are tracked in time directly.

  - No exact closure, instead just numerical quadrature

- Moment Evolution Equations

  - For n-point quadrature, the moments are written as:

    - μk=rkfrdr=i=1nrikwi

    - For k = 0 through 2*n – 1

- Solve using a tridiagonal Jacobi matrix to find the abscissas and weights.  Appendix A provides algorithm for calculating the weights and absissas

Other topics

Gaussian Quadrature

- Doesn’t evaluate at evenly spaced points, instead finds the best possible points and weights to approximate polynomials

- Can be exact for polynomials up to order 2*N-1, where N is number of points

- Uses different base polynomials: Legendre, Laguerre, hermite, Chebyshev

### Distribution Reconstruction

John, V., I. Angelov, A.A. Öncül, and D. Thévenin. “Techniques for the Reconstruction of a Distribution from a Finite Number of Its Moments.” Chemical Engineering Science 62, no. 11 (June 2007): 2890–2904. https://doi.org/10.1016/j.ces.2007.02.041.

Notes:

- Methods evaluated: Prescribed functions, discrete method, spline-based reconstruction

- Even knowing all the moments is not sufficient to determine a function uniquely

- Instead, a priori restriction  of function class is required

  - “Following this guideline, chemical engineers dealing with applications generally assume a very simple a priori shape for the distribution (Gaussian, log-normal, -function, etc.), so that only a very small number of moments are needed to determine the best fit within this assumed class of functions.”

## Moving Pivot Technique

Kumar, Sanjeev, and D. Ramkrishna. “On the Solution of Population Balance Equations by Discretization—II. A Moving Pivot Technique.” Chemical Engineering Science 51, no. 8 (April 1996): 1333–42. https://doi.org/10.1016/0009-2509(95)00355-X.

Notes:

- Goal is to account for the variation of number densit with size within a size range used for discretization

- Selection of pivot location within a size range iss chosen to reflect variation of the density in the range

- “For the type of density shown in Fig. 1, x~ stays closer to the middle of the section and for steeply decreasing densities, as shown in Fig. 2, it stays closer to the lower end of the section, i.e. closer to v~, thus capturing the aggregation process more realistically. If the density in a section changes from steeply decreasing to nearly uniform due to the particulate process, the pivot moves from the lower end of the section to the middle.”

- Discrete equations: